Assistive EOG Blink Switch

| Gesture | Interface Role | Example Use Case |
| --- | --- | --- |
| Single Blink | Navigation (Scan) | Moving to the next letter on a screen, or selecting the next option in main menu, or cycling to the next room appliance |
| Double Blink | Selection (Execute) | Pressing "Enter", or toggling the selected switch on/off. |
| Triple Blink | Cancel / Back | Deleting a typed letter, or returning to the main menu. |
| Long Hold (3s+) | Emergency Override | Squeezing eyes shut to sound an alarm or call a caregiver. |

### 1. Digital Communication (AAC)

We can host a simple web page directly on the ESP32 that displays a grid of letters or common phrases. The system automatically highlights rows one by one. A single blink skips ahead, and a double blink selects the highlighted letter to build a sentence, which the web browser can then read aloud.

2. Smart Room Automation

We can give the user direct control over their physical environment without needing a smartphone. By mapping a double-blink to an ESP32 digital output pin, we can trigger a standard relay or switch high-power loads to turn on a desk lamp, activate a heater, or control a motorized fan.

### 3. Actuating Physical Mechanisms

Blinks can be translated into mechanical assistance. We can use the ESP32's PWM capabilities to physically unlock a door latch, dispense medication from a hopper etc when a specific blink sequence is registered.

### 4. Emergency Alerts

Safety features will override all other modes. A "Long Hold" (keeping eyes squeezed shut for three seconds) creates a massive, sustained voltage spike that is impossible to trigger by accident. We can map this to trigger a loud buzzer, or use the ESP32's WiFi to send an automated webhook to Telegram or Whatsapp or make an emergency call, instantly alerting a caregiver's phone.

Note: We can combine all these together in a website, and the user can select what function he wants to use.