Node 1: The Wearable Headband (Transmitter & Web Server)

- 1x ESP32 Microcontroller: standard ESP32 NodeMCU to process blinks and host the Wi-Fi dashboard. 350/-

- 1x BioAmp EXG Pill: The analog front-end board to amplify the EOG signals. 2599/-

- 1x BioAmp Cable: To connect the sensors to the board.  included with EXG pill

- 1x Elastic Sports Sweatband: The physical mount for the wearable. ?

- 2x Metal Snap Buttons: Standard clothing snaps pushed through the sweatband to act as the flat, dry IN+ and IN- forehead electrodes.  already have

- 1x EEG Ear Clip Electrode: A reusable clip for the REF (reference) connection on the earlobe. 399/-

- 1x 3.7V LiPo Battery: A small, lightweight battery (e.g., 500mAh - 1000mAh) to power the headset safely. 500-700/-

- 1x 10µF to 100µF Electrolytic Capacitor: Placed across the BioAmp's power pins to absorb ESP32 Wi-Fi current spikes and keep the analog signal clean. Already have

- 2x 100kΩ Resistors: Used to build the voltage divider for safely reading the LiPo battery level. already have

Node 2: The Room Control Hub (Receiver & Internet Gateway)

- 1x ESP32 Microcontroller: A standard ESP32 development board to receive ESP-NOW packets and connect to the router.

- 1x 5V Optocoupled Relay Module (1 or 2 Channels): For safely switching 230V AC mains appliances like a desk lamp or room fan.

- 1x MG995 Servo Motor: A high-torque actuator for mechanical tasks like pulling a door latch or operating a dispenser.

- 1x Active Piezo Buzzer: To sound the loud localized alarm when an emergency long-hold blink is detected.

- 1x 5V USB Wall Adapter & Cable: To provide constant wall power to the hub.

Miscellaneous

- Jumper wires and perfboard: For assembling the circuits.

- Saline solution or aloe vera gel: To dab onto the metal snap buttons right before wearing to drop the skin impedance.

- A Tablet, Smartphone, or Smart TV: To connect to the Headband's Access Point and display the HTML/JS scanning dashboard.